Project Title:

Scalable Enterprise Network Design

Submitted by:

233576    MUSA BIN ABDULLAH

233588    M. DANIAL

233550    AMMAR YAQUB

Group No:

09

Submitted to:

Sir Abdul Ghani

Contents:

Project Report: Jackson Company Network Infrastructure	2

1. Introduction	3

2. VLAN Design & Subnetting	3

Sydney Branch	3

Melbourne Branch	4

Brisbane Branch	4

Hong Kong Branch	4

3. DHCP Configuration	5

4. Internal Server Configuration	5

5. Routing Configuration	6

6. ACL Firewall Rules	6

7. Device Security Configuration	6

8. Wireless & VoIP	7

9. Testing & Verification	7

# Project Report: Jackson Company Network Infrastructure

## 1. Introduction

This report presents the comprehensive network infrastructure design and configuration for Jackson Company's branches located in Sydney, Melbourne, Brisbane, and Hong Kong.

- Student ID: 233588

- VLAN segmentation for department isolation

- Subnetting using Variable Length Subnet Masking (VLSM)

- DHCP configuration for client devices

- Static and OSPF-based dynamic routing

- Internal servers for DNS and HTTP

- ACL-based firewall for security

- Basic device hardening

- Wireless access points and VoIP integration

- Final testing using simulation tools

The IP address space allocated for this project is 188.88.0.0/16, based on the student ID. All addressing schemes are derived from this base using VLSM for optimal usage.

## 2. VLAN Design & Subnetting

### Sydney Branch

| VLAN | Department | Subnet | Gateway |
| --- | --- | --- | --- |
| 10 | HR | 188.88.0.0/21 | 188.88.0.1 |
| 20 | IT | 188.88.8.0/21 | 188.88.8.1 |
| 30 | Finance | 188.88.20.0/21 | 188.88.20.1 |
| 40 | Admin | 188.88.16.0/22 | 188.88.16.1 |
| 50 | Servers | 188.88.36.0/25 | 188.88.36.1 |
| 60 | Guest WiFi | 188.88.32.0/21 | 188.88.32.1 |

### Melbourne Branch

| VLAN | Department | Subnet | Gateway |
| --- | --- | --- | --- |
| 10 | HR | 188.88.64.0/19 | 188.88.64.1 |
| 20 | IT | 188.88.96.0/23 | 188.88.96.1 |
| 30 | Finance | 188.88.98.0/23 | 188.88.98.1 |
| 40 | Admin | 188.88.100.0/23 | 188.88.100.1 |
| 50 | Servers | 188.88.102.0/25 | 188.88.102.1 |
| 60 | Guest WiFi | 188.88.104.0/23 | 188.88.104.1 |

### Brisbane Branch

| VLAN | Department | Subnet | Gateway |
| --- | --- | --- | --- |
| 10 | HR | 188.88.76.0/20 | 188.88.76.1 |
| 20 | IT | 188.88.80.0/23 | 188.88.80.1 |
| 30 | Finance | 188.88.82.0/23 | 188.88.82.1 |
| 40 | Admin | 188.88.84.0/23 | 188.88.84.1 |
| 50 | Servers | 188.88.86.0/25 | 188.88.86.1 |
| 60 | Guest WiFi | 188.88.88.0/23 | 188.88.88.1 |

### Hong Kong Branch

| VLAN | Department | Subnet | Gateway |
| --- | --- | --- | --- |
| 10 | HR | 188.88.96.0/23 | 188.88.96.1 |
| 20 | IT | 188.88.100.0/23 | 188.88.100.1 |
| 30 | Finance | 188.88.102.0/23 | 188.88.102.1 |
| 40 | Admin | 188.88.104.0/23 | 188.88.104.1 |
| 50 | Servers | 188.91.36.0/25 | 188.91.36.1 |
| 60 | Guest WiFi | 188.88.108.0/23 | 188.88.108.1 |

## 3. DHCP Configuration

DHCP services are enabled for all client-facing VLANs using Layer 3 switches. Static reservations are used for servers. DNS settings for each branch:

| Branch | DNS Server IP |
| --- | --- |
| Sydney | 188.88.36.12 |
| Melbourne | 188.88.64.12 |
| Brisbane | 188.88.90.12 |
| Hong Kong | 188.88.106.12 |

## 4. Internal Server Configuration

Each branch hosts 6 core servers on VLAN 50:

| Service | Sydney | Melbourne | Brisbane | Hong Kong |
| --- | --- | --- | --- | --- |
| DNS Server | 188.88.36.12 | 188.88.102.12 | 188.88.90.12 | 188.91.36.12 |
| HTTP Server | 188.88.36.11 | 188.88.102.11 | 188.88.90.11 | 188.91.36.11 |
| FTP Server | 188.88.36.13 | 188.88.102.13 | 188.88.90.13 | 188.91.36.13 |
| Mail Server | 188.88.36.14 | 188.88.102.14 | 188.88.90.14 | 188.91.36.14 |
| Database Server | 188.88.36.15 | 188.88.102.15 | 188.88.90.15 | 188.91.36.15 |
| Backup Server | 188.88.36.16 | 188.88.102.16 | 188.88.90.16 | 188.91.36.16 |

## 5. Routing Configuration

All routers use OSPF for dynamic routing. Example: Sydney Router (R1)

- G0/0: 188.88.254.1 (to Core Switch)

- G0/1: 10.0.0.1 (to Melbourne)

OSPF Configuration:

router ospf 1

network 188.88.0.0 0.0.255.255 area 0

network 10.0.0.0 0.0.0.3 area 0

## 6. ACL Firewall Rules

Access-list applied to Server VLANs to restrict Guest VLAN access.

Example (Sydney):

ip access-list extended SERVERS_PROTECT

deny ip 188.88.32.0 0.0.7.255 188.88.36.0 0.0.0.127

permit ip any any

## 7. Device Security Configuration

- Hostnames configured

- Console password: class123

- VTY password: remote123

- Enable secret: cisco123

- Service password-encryption enabled

- Banner: Unauthorized access is prohibited!

- IP domain lookup disabled

## 8. Wireless & VoIP

- Access Points deployed in each branch for wireless clients

- Devices include , smartphone, and laptop

- IP phones configured per department

## 9. Testing & Verification

- DHCP IP allocation

### Sydney Branch

Hr

IT

Finance

Admin

### Melbourne Branch

Hr

IT

Finance

Admin

### Brisbane Branch

Hr

IT

Finance

Admin

### Hong Kong Branch

Hr

IT

Finance

Admin

- DNS Resolution

### Sydney Branch

### Melbourne Branch

### Brisbane Branch

### Hong Kong Branch

- HTTP Access

### Sydney Branch

### Melbourne Branch

### Brisbane Branch

### Hong Kong Branch

- Inter-Branch Ping

Melbourne to Sydney

Sydney to Melbourne

Melbourne to Brisbane

Brisbane to Melbourne

Brisbane to Hong Kong

Hong Kong to Brisbane

- Banner on routers

### Sydney Branch

### Melbourne Branch

### Brisbane Branch

### Hong Kong Branch